Intel® Virtual RAID on CPU (Intel® VROC)

7.7 PV Windows Customer Release Notes

November 2021

- Revision History

| External Version | Internal Version | Description | Date |
| --- | --- | --- | --- |
| 001 | 1.0 | Initial Intel VROC 6.3 PV Release. | February 202 |
| 002 | 1.1 | Initial Intel VROC 7.0 PV Release. | July 2020 |
| 003 | 1.2 | Intel VROC 7.0 PV Release to address Potential Data Corruption Condition | August 2020 |
| 004 | 1.3 | Initial Intel VROC 7.5 PV Release. | February 2021 |
| 005 | 1.4 | 7.6 PC candidate for Idaville LCC/HCC | April 2021 |
| 006 | 1.5 | 7.6 PV Maintenance Build Release package | April 2021 |
| 007 | 1.6 | 7.7 PV Maintenance Release Package | November 2021 |

Intel may make changes to specifications and product descriptions at any time, without notice. Designers must not rely on the absence or characteristics of any features or instructions marked "reserved" or "undefined". Intel reserves these for future definition and shall have no responsibility whatsoever for conflicts or incompatibilities arising from future changes to them. The information here is subject to change without notice. Do not finalize a design with this information.

No computer system can provide absolute security. Requires an enabled Intel® processor, enabled chipset, firmware and/or software optimized to use the technologies. Consult your system manufacturer and/or software vendor for more information.

Intel technologies’ features and benefits depend on system configuration and may require enabled hardware, software or service activation. Performance varies depending on system configuration. Check with your system manufacturer or retailer or learn more at intel.com.

The products described in this document may contain design defects or errors known as errata which may cause the product to deviate from published specifications. Current characterized errata are available on request.

Contact your local Intel sales office or your distributor to obtain the latest specifications and before placing your product order.

All products, computer systems, dates, and figures specified are preliminary based on current expectations, and are subject to change without notice.

© Intel Corporation. Intel, the Intel logo, and other Intel marks are trademarks of Intel Corporation or its subsidiaries. Other names and brands may be claimed as the property of others.

Revision History	2

1	Introduction	5

1.1	Terminology	5

1.2	Defect Submission Support	7

1.3	Supported Operating Systems	8

1.4	Supported Platforms/Chipsets/SKUs	9

2	Supported PCIe NVMe SSDs List	11

3	New Features	12

3.1	New Features Introduced with Intel VROC 7.7	12

3.2	New Features Introduced in the Intel VROC 7.6 Release	13

3.3	New Features in Intel VROC 7.5 PV	13

3.4	Critical Issue Resolved in Intel VROC 7.0.2 PV	15

3.5	New Features in Intel VROC 7.0 PV	16

4	Drivers, Images and Utilities	17

5	Intel VROC Limitations	19

5.1	Microsoft .NET Framework Removal	19

5.2	Surprise Hot Plug Limitations	20

5.3	Expect Longer Rebuild Times for RAID 5	20

5.4	Intel VROC Command Line Interface	20

5.5	Intel VROC Trial Version Limitations	20

5.6	Intel VROC PreOS UEFI Driver Uninstall limitations	20

5.7	Intel NVMe Wear Leveling Recommendations	20

5.8	Must use F6 Install Method	22

5.9	Intel C620 and C422 series chipset Port Limitations	22

5.10	Intel VROC Key Removal/Upgrade Limitation	22

5.11	NVMe Port Assignment by Intel VROC	23

5.12	Windows 10 RS5/Server 2019	23

5.13	Intel VROC 6.X on Windows Server 2012 R2	23

5.14	Intel VROC (NonVMD NVMe RAID) Upgrade	23

5.15	Intel VMD Support on 8 Socket Platforms	24

5.16	Intel VROC SATA LED Management	24

5.17	Intel VROC Creation Volume Sizes	24

5.18	MCERR/P_CATERR/Bus Uncorrectable Error with Intel VMD enabled	24

5.19	Intel VROC Legacy Option ROM (OROM)	24

6	Intel VROC Issues	25

6.1	Intel VROC Known Issues	25

6.2	Resolved Issues in Intel VROC 7.7 PV	26

6.3	Resolved Issues in Intel VROC 7.6 PV	28

6.4	Resolved Issues in Intel VROC 7.5 PV	29

6.5	Resolved Issues in Intel VROC 7.0.2 PV	30

6.6	Resolved Issues in Intel VROC 7.0 PV	31

6.7	Resolved Issues in Intel VROC 6.3 PV	32

6.8	Resolved Issue in Intel VROC 6.2 PV	33

# Introduction

The Intel® Virtual RAID on CPU (Intel® VROC) 7.7 PV release is a family of products providing enterprise RAID solutions targeted to support Whitley Ice Lake based platforms.

- Intel VROC (VMD NVMe RAID) provides enterprise RAID solution on platforms that supports Intel® Volume Management Device (Intel® VMD) on Intel® Xeon® Scalable Processors.

- Intel VROC (SATA RAID) provides an enterprise RAID solution for SATA devices connected to SATA/sSATA ports of the Intel® Platform Controller Hub (Intel PCH) configured for RAID Mode.

- Intel VROC (NonVMD NVMe RAID) – This product provides an enterprise RAID solution for Intel NVMe SSDs attached to PCIe slots managed by the Platform CPU.  Intel VROC (NonVMD NVMe RAID) is not intended for, nor supports:

  - Non-Intel NVMe SSDs.

  - Platforms that have Intel CPUs that contain Intel VMD technology (weather enabled or disabled).

Intel VROC is a high-level blanket product reference for Intel VROC (VMD NVMe RAID), Intel VROC (SATA RAID) and Intel VROC (NonVMD NVMe RAID).

Intel VROC 7.7 PV Release is a high-level blanket product reference for the family of products that include Intel VROC (VMD NVMe RAID) and Intel VROC (SATA RAID).

Please see the Supported Platforms section for additional information on older platforms supported with this release.

It is always recommended to update your system BIOS to the included PV release of PreOS images to take advantage of the most optimal and updated features of each Production Version release

These three products are provided in a single product package, can be installed, and used independently.  For example (unless otherwise restricted by the platform itself), Intel VMD is not required to enable and use Intel VROC (SATA RAID).  Likewise, Intel VROC (VMD NVMe RAID) can be enabled and used without Intel VROC (SATA RAID).

## Terminology

Table 1:	Terminology

| Term | Description |
| --- | --- |
| AHCI | Advanced Host Controller Interface |
| API | Application Programming Interface |
| ASM | Intel® Accelerated Storage Manager (Intel® ASM) |
| BIOS | Basic Input/Output System |
| GB | Gigabyte |
| GUI | Graphical User Interface |
| HII | Human Interface Infrastructure |
| Hot-Plug | The unannounced removal and insertion of a drive while the system is powered on. |
| I/O | Input/Output |
| KB | Kilobyte |
| Matrix RAID | Two independent RAID volumes within a single RAID array. |
| MB | Megabyte |
| Member Disk | An NVMe drive used within a RAID array. |
| NVMe | Non-volatile Memory Express |
| OS | Operating System |
| POST | Power On Self-Test |
| PreOS | The Intel VROC images incorporated into the platform BIOS to access the drives and providing the interface to configure Intel VROC UEFI Drivers. |
| RAID | Redundant Array of Independent Disks: allows data to be distributed across multiple drives to provide data redundancy or to enhance data storage performance. |
| RAID 0 (striping) | The data in the RAID volume is striped across the array's members. Striping divides data into units and distributes those units across the members without creating data redundancy but improving read/write performance. |
| RAID 1 (mirroring) | The data in the RAID volume is mirrored across the RAID array's members. Mirroring is the term used to describe the key feature of RAID 1, which writes duplicate data from one drive to another; therefore, creating data redundancy and increasing fault tolerance. |
| RAID 5 (striping with parity) | The data in the RAID volume and parity are striped across the array's members. Parity information is written with the data in a rotating sequence across the members of the array. This RAID level is a preferred configuration for efficiency, fault-tolerance, and performance. |
| RAID 10 (striping and mirroring) | The RAID level where information is striped across two drive arrays for system performance. Each of the drive in the array has a mirror for fault tolerance. RAID 10 provides the performance benefits of RAID 0 and the redundancy of RAID 1. However, it requires four hard drives so it’s the least cost effective. |
| RAID Array | A logical grouping of physical drives. |
| RAID Volume | A fixed amount of space across a RAID array that appears as a single physical drive to the operating system. Each RAID volume is created with a specific RAID level to provide data redundancy or to enhance data storage performance. |
| Spare | The drive that is the designated target drive in a RAID Volume recovery. The Spare drive is a global setting (not designated to a specific RAID volume). Spare drives on a SATA Controller are not available on the sSATA Controller (and visa-versa). Spare drives designated on Intel VROC (VMD NVMe RAID) are exposed and available on all Intel VMD domains. |
| Strip | The size of the data block that is to be written in each write cycle across the RAID array. |
| Stripe | Block size that is assigned to evenly distribute portions of the stripe across a designated number of drives within a RAID array. A collection of Strips is called a Stripe |
| Intel® RSTe | Intel® Rapid Storage Technology enterprise. |
| RWH | RAID Write Hole |
| SSD | Solid State Drive |
| TB | Terabyte |
| UEFI Mode | Unified Extensible Firmware Interface. Refers to the system setting in the BIOS |
| Intel® VMD | Intel® Volume Management Device |
| Intel® VROC | Intel® Virtual RAID on CPU |

## Defect Submission Support

With this release, Intel will accept, and process issues reported by customers via the Intel Premier Support (IPS) portal.

To submit an issue, please use the Intel Premier Support (IPS) tool. Information, training and details can be found at the below website. Your local Intel FAE can also provide you the necessary requirements to enable you to submit an IPS issue (also known as a “case”) including an account setup if you do not already have one.

http://www.intel.com/content/www/us/en/design/support/ips/training/welcome.html

When submitting a case, please include the following Fields in order to flag Intel VROC AE support for Intel® Xeon® Scalable platforms.

- Case Information -> Product = Purley/Whitley/EagleStream

- Case Details -> Subject= <Add short title summary of issue>

- Case Details -> Case Description = <add description and how to reproduce error)

- Case Details -> Case Type = <fill in type of request>

- Case Details -> Severity = <fill in severity of issue>

- Case Details -> End Customer = <name of OEM>

- Case Details -> Issue Source = IPS Cloud

- Case Details -> Severity

- Product/Project Info -> Case Category = TechnologyInitiative

- Product/Project Info -> Case Subcategory = Intel® Rapid Storage Technology enterprise (Intel® RSTe) 
OR
Product/Project Info -> Case Subcategory = Intel® Virtual RAID on CPU (Intel® VROC)

- Environment Details -> Purley-PCH = lbg-4 (select from appropriate options available)

- Environment Details -> Purley-CPU = skx-2s (or skx 4s) (select from appropriate options available)

- Environment Details -> BKC or SW Version = <applicable versioning information>

## Supported Operating Systems

The Production Version (PV) release package for the Intel VROC 7.7 family of products was designed to work with, tested and validated on the following Windows OSs.

| Platform† | Windows10 (RS3, RS4, RS5††, 19H1, 19H2, 20H1, 20H2, 21H1, 21H2), | Windows 11 | Windows Server 2016 RS4 Enterprise | Windows Server 2019 Enterprise (19H1, 19H2††, 20H1, 20H2, 21H1, 21H2) | Windows Server 2022 Enterprise |
| --- | --- | --- | --- | --- | --- |
| #### Intel® C610/C620 series chipset based platforms | #### Y | #### Y | #### Y | #### Y | #### Y |
| #### Intel® C220/C230/C240 series chipset based platforms | #### Y | #### Y | #### Y | #### Y | #### Y |
| #### Intel® Xeon® Scalable Processor family based platforms | #### Y | #### Y |  |  |  |
| #### Intel® Xeon® Scalable Processor family Workstation based platforms | #### Y | #### Y |  |  |  |
| #### Intel® Xeon® Scalable Processor family Server based platforms |  |  | #### Y | #### Y | #### Y |
| #### Intel® Xeon® Processor D-2100 Product Family based platform |  |  | #### Y | #### Y | #### Y |

- †Only 64bit OS is supported on all platforms
†† Introduces/Includes WinPE Support for this Version

## Supported Platforms/Chipsets/SKUs

The Intel VROC 7.7 PV package was designed to work with, tested and validated on Intel Customer Reference Boards (CRBs) outlined in this section.

### Supported Platforms for Intel VROC (VMD NVMe RAID)

| CPU | Platform | VMD 1.0 Device ID | VMD 2.0 Device ID | # of VMD |
| --- | --- | --- | --- | --- |
| #### Intel® Xeon® Scalable Processor family – W | #### Intel® Xeon® Scalable Processor family workstation† | #### 201D | #### N/A | #### 3 per CPU |
| #### Intel® Xeon® Scalable Processor family – SP | #### Intel® Xeon® Scalable Processor family server and workstation† | #### 201D | #### 28C0 | #### 3 per CPU |
| #### Intel® Xeon® Processor D-2100 Product Family | #### Intel® Xeon® Processor D-2100 Product Family based platform† | #### 201D | #### N/A | #### 3 per CPU |
| #### The Intel® Xeon® Processor Scalable Memory Family | #### Intel® Xeon® Scalable Processor family server† | #### 201D | #### 28C0 | #### 4 per CPU (VMD 1.0) 5 per CPU (VMD 2.0) |
| #### Intel® X299 High End Desktop | #### Intel® Xeon® Scalable Processor family server and workstation† | #### 201D | #### N/A | #### 3 per CPU |

† Unless Otherwise Specified in the Release Notes

Intel VROC (VMD NVMe RAID) support on the Intel X299 High End Desktop platforms is restricted to Intel NVMe SSDs only. This is a platform limitation. As a result, when Intel VROC is installed onto an Intel X299 High End Desktop platform, the customer will only see Intel NVMe SSDs plugged into the platform.

Intel VROC (NonVMD NVMe RAID) is not supported on platforms that support Intel VMD (enabled or disabled).

### Supported Chipset SKU for Intel VROC (SATA RAID)

| Chipset | Platform | RAID controller Device ID | Intel VROC (NonVMD NVMe RAID) Virtual Device ID | # of ports |
| --- | --- | --- | --- | --- |
| #### Intel® C610 series chipset | #### Platforms containing the Intel® C610 series chipset† | #### 2826 (SATA) 2827 (sSATA) | #### 2F9c | #### 6 SATA 4 sSATA |
| #### Intel® C610 series chipset | #### Platforms Refreshes containing the Intel® C610 series chipset† | #### 2826 (SATA) 2827 (sSATA) | #### 6F9C | #### 8 SATA 6 sSATA |
| #### Intel® C220 series chipset | #### Platforms containing the Intel® C220 series chipset† | #### 2826 (SATA) | #### N/A | #### 6 SATA |
| #### Intel® C230 series chipset | #### Platforms containing the Intel® C230 series chipset Platform† | #### 2826 (SATA) | #### A135 (Integrated Sensor Hub enabled) | #### 8 SATA |
| #### Intel® C240 series chipset | #### Platforms containing the Intel® C240 series chipset† | #### 2826 (SATA) | #### A37C (Integrated Sensor Hub enabled) | #### 8 SATA |
| #### Intel® 620 series chipset | #### Platforms containing the Intel® 620 series chipset† | #### 2826 (SATA) 2827 (sSATA) | #### N/A | #### 8 SATA 6 sSATA |
| #### Intel® C422 series chipset | #### Platforms containing the Intel® C422 series chipset† | #### 2826 (SATA) | #### N/A | #### 8 SATA |

† Unless otherwise specified in the Release Notes

# Supported PCIe NVMe SSDs List

All shipping Intel® Data Center and Professional NVMe* SSDs are supported by Intel® VROC 7.7 PV, except dual port NVMe* SSDs.  For the latest list of supported Non-Intel PCIe NVMe SSDs, please visit Intel® Virtual RAID on CPU (Intel® VROC) Supported Configurations

Platform providers are now allowed to self-validate their own list of NVMe SSDs for use with Intel VROC (VMD NVMe RAID). For more details, please contact your designated Intel VROC PAE.

# New Features

## New Features Introduced with Intel VROC 7.7

### UEFI Firmware Management Protocol Support

Intel VROC 7.7 introduces limited support for the UEFI Firmware Management Protocol (FMP) as outlined in UEFI Specification version 2.9.  The Intel VROC UEFI drivers will provide support for updating drive firmware (through UEFI FMP) when the drive is managed by Intel VROC.  This includes both Intel VROC (VMD NVMe RAID) and Intel VROC (SATA RAID).  The Intel VROC UEFI driver has implemented a subset of protocol functions defined in the UEFI Specification, which is reflected in the below table.

| EFI Firmware Management Protocol functions | Intel VROC UEFI implementation |
| --- | --- |
| GetImageInfo | Supported |
| SetImage | Supported |

The following diagram highlights the process flow to support both NVMe and SATA drives.

### Intel VROC (SATA RAID) Support of EFI_ATA_PASS_THRU_Protocol

The Intel VROC 7.7 release package introduces limited support for EFI_ATA_PASS_THRU_PROTOCOL commands to provide information on the SATA drives managed by Intel VROC (SATA RAID).

### Disable Locate LED within Intel VROC LED Management

The Intel VROC provides support for the OEM/ODMs to disable the Locate LED functionality within the Intel VROC LED Management.  By enabling the feature, the OEM/ODM can use their own tools to initiate a Locate LED functionality within their platform.

## New Features Introduced in the Intel VROC 7.6 Release

The Intel VROC 7.6 release package introduces two new features.  These two features are Intel VROC PreOS Failed RAID Volume (limited) recovery as well as UEFI ATA Passthrough protocol support for Intel VROC (SATA RAID)

### Intel VROC PreOS Environment RAID Volume Failure Recovery

Intel VROC 7.6 PV introduces a new feature in the VROC HII page that will allow the user to attempt to recover from a failed RAID volume.  When a failed RAID volume is encountered during boot, the option is made available in the VROC HII page for which the user can select.  This is a multi-step process.

The first is to select the RAID volume in “Failed” state and then select the option to force it to “Degraded” state.  This will expose the next option to specify the drive to enable this action.

Once the RAID volume is changed to “Degraded” state, the standard RAID volume recovery process initiates the rebuilding process.

NOTE:  Intel makes no guarantee of successful recovery from a failed state using this option. This must be treated as a last chance effort and there is no guarantee that there won’t be some data loss. Intel always recommends recovering a Failed RAID Volume by recreating the RAID volume from scratch and restore the data from the latest platform image backup.

### Intel VROC (SATA RAID) UEFI Support for  EFI_ATA _PASSTHRU

Intel VROC 7.6 VROC PV introduces support for EFI_ATA_PASS_THRU protocol support by the Intel VROC (SATA RAID) UEFI driver.  The following specific options are supported in Intel VROC 7.6:

- EFI_ATA_PASS_THRU_PASSTHRU PassThru;

  - IDENTIFY

  - ATA_READ_LOG_EXT – There is at least one known bug currently being worked on.

- EFI_ATA_PASS_THRU_GET_NEXT_PORT GetNextPort;

- EFI_ATA_PASS_THRU_GET_NEXT_DEVICE GetNextDevice;

- EFI_ATA_PASS_THRU_BUILD_DEVICE_PATH BuildDevicePath;

## New Features in Intel VROC 7.5 PV

### Increasing MSIX Vectors to 64

Intel VROC 7.5 introduces support for customer configurations that can support 64 MSIX Vectors.   On platforms that support Intel VMD 1.0, the MSIX support is limited to 32 MSIX vectors.  For these (Intel VMD 1.0) platforms, a single VMD domain can support up to 24 NVMe SSDs.  This means that those 24 NVMe SSDs will share a single Intel VMD MSIX vector.  As the number of vectors, in newer NVMe devices, increase beyond 32, this can result in a platform performance impact.  With the introduction of Intel VMD 2.0, and Intel VROC 7.5, this increase to 64 MSIX vectors should help to alleviate this problem.   This is because the average dual socket server will have between 48 and 64 cores, which limits the number of MSIX vectors on a given NVMe SSD to 64, for optimal usage.

### Intel VMD (PCH) Support

Intel VROC 7.5 introduces support for the Intel (VMD NVMe RAID) management of NVMe SSDs connected to the Platform Controller Hub (PCH).  Utilizing the Flex-IO capabilities of the PCH, Intel VMD 2.0 technology can now take ownership of two of the slots allowing Intel VROC (VMD NVMe RAID) to control and manage the NVMe SSDs attached to those slots.  This is accomplished by utilizing the Function Level Assignment of the PCH PCI functions.  Utilizing the BIOS setup menus, the user will be able to enable Intel VMD on the designated slots, or PCH functions (depending on the BIOS implementation) and the PCH functions will be reassigned to an Intel VMD (PCH) controller.  To be able to accomplish this, there must be NVMe SSDs present on the slot(s) and the slots used must support Slot Implemented Capabilities.  Otherwise, Intel VMD (PCH) cannot be enabled.  Please refer to the Intel® C620 Series Chipset Platform Controller Hub datasheet (document No. 336067) and/or the Ice Lake PCH External Design Guide, document No. 572631 for details on the Flexible I/O feature.

When this feature is fully enabled, it will reassign sSATA ports 2-5 to be PCIe lanes managed by Intel VMD (PCH).  This is translated into PCIe root ports 8-11 and Flex I/O ports 14-17. The general configuration is 2 - X2 PCIe lanes.

This feature is intended for supporting a simple RAID 1 boot using 2- NVMe SSDs attached directly to the PCH.  However, if the platform supports expanded configurations, using retimer or switch Add-In-Cards (AIC), full Intel VROC (VMD NVMe RAID) support can be obtained.  Intel VROC (VMD NVMe RAID) was only validated with 2 NVMe SSDs directly attached to the PCH.  Any configurations beyond two directly attached NVMe drives is not recommended.

With this new functionality, when Intel VMD is enabled, a DUMMY function/device will be seen in the Windows Device Manager as a “Yellow Bang”.  This function is the result of a PCIe requirement for a device with multiple functions. Once the Intel VROC installation process is complete this DUMMY function/device will be hidden in the list of System functions.

#### Intel VMD (PCH) PreOS Support

Intel VMD (PCH) PreOS support is included in the Intel VROC (VMD NVMe RAID) PreOS images, which is part of the Intel VROC release package.  There are no other PreOS images required.

#### Intel VMD (PCH) Pass-thru Boot Support

Intel VROC 7.5 provides Intel VROC (VMD NVMe RAID) PreOS support for NVMe SSDs attached to the PCH when Intel VMD (PCH) is enabled.  This will allow an OS to be installed onto and boot from an NVMe device managed by Intel VROC (VMD NVMe RAID).  No Intel VROC Upgrade key is required to utilize this feature.

#### Intel VMD (PCH) RAID Boot Support

Intel VROC 7.5 provides Intel VROC (VMD NVMe RAID) PreOS support for NVMe SSDs attached to the PCH when Intel VMD (PCH) is enabled.  When an Intel VROC Upgrade key is present and Intel VMD (PCH) is enabled, the user will be able to use the Intel VROC (VMD NVMe RAID) PreOS HII to setup and manage a RAID volume using the NVMe SSDs attached to the PCH.  This will allow an OS to be installed onto and boot from an Intel VROC (VMD NVMe RAID) volume attached to Intel VMD (PCH).

#### Intel VMD (PCH) RAID Data Volume Spanning

Once Intel VMD (PCH) is enabled, Intel VROC (VMD NVMe RAID) will treat it like any other Intel VMD controller.  Spanning Data RAID volumes are supported as they are currently outlined in this document.  This applies to the Intel VROC PreOS tools, the Windows OS GUI and the CLI tool.  The option to span VMD controllers is supported, but not recommended

This is supported but not recommended due to a performance penalty using the PCH.

Windows boot volumes cannot span between PCH and CPU VMD devices. Meaning all NVMe SSDs in the boot array, must be on the VMD (PCH) device.

#### Intel VMD (PCH) Designations

The Intel VROC (VMD NVMe RAID) management tools (PreOS tools, Windows GUI and CLI tool) are used to manage devices attached to the Intel VMD (PCH) controller, the corresponding information displayed will indicate “PCH” with any device or controller associated with Intel VMD (PCH).

#### Intel VMD (PCH) Hot Plug Support.

Intel VROC does not support Hot Plug when NVMe SSDs are attached to the Intel VMD (PCH) controller.

### Number of Intel VMD Increased

Intel VMD 2.0 technology and increased the number of PCIe lanes controlled by the Intel VMD from 48 to 64.  This, along with the Intel VMD (PCH) support increases the total number of Intel VMDs to 5.

- 4 Intel VMDs off the CPU

- 1 Intel VMD of the PCH (Intel VMD (PCH))

### Native PCIe Enclosure Management (NPEM)

Intel VROC 7.5 introduces support for the Native PCIe Encloser Management (NPEM) standard for LED management in a PCIe 4.0 based environment.   This capability is discoverable in each switch-downstream-port.  If it is discovered to be present, the Intel VROC LED utility will use NPEM control, capability and status registers to visually indicate the various drive and volume states.

## Critical Issue Resolved in Intel VROC 7.0.2 PV

A potential silent data loss condition exists in both the Intel VROC (SATA RAID) and the Intel RSTe windows-based products. The potential silent data loss condition exists when the Intel VROC Read Patrol feature is enabled for redundant SATA RAID volumes (RAID1, 5 or 10) and one of the RAID member drives is found to have a Bad Block condition. The Read Patrol process, of using the redundant data to correct the Bad Block, can result in invalid data being written.  The silent data loss exposure must include all the following conditions:

- The Intel VROC Read Patrol feature is enabled

- A redundant SATA RAID volume present

- One of the SATA RAID member drives has a Bad Block.

If any one of the above conditions is not present, the silent data loss does not occur.  Nor does this condition exist on Intel VROC (VMD NVMe RAID).

## New Features in Intel VROC 7.0 PV

There are no new features included in the Intel VROC 7.0 PV release package.

### AHCI Driver Support Removed

With the release of Intel VROC 7.0, the package will no longer include an Intel VROC AHCI driver to support the PCH configured for AHCI mode.  The AHCI driver included in the Microsoft OS distribution will be the best option to support this configuration.

The Intel VROC GUI will not display any SATA drives that are managed by the SATA controller in AHCI mode.

### Microsoft Windows 7 support Removed

With the release of Intel VROC 7.0, the package will no longer include support for Microsoft Windows 7.  If this type of support is required, please refer to the Intel VROC 6.3 PV package available on VIP.

### Legacy Option ROM Support Removed

With the release of Intel VROC 7.0, the package will no longer include Legacy Option ROM images for the PCH Controller (in RAID Mode).  For older platforms that may still require Legacy Option ROM support, please refer to the Intel VROC 6.3 PV package to obtain those images.  Intel VROC 7.0 will support environments that have the Legacy Option ROM images from the Intel VROC 6.3 PV package.

# Drivers, Images and Utilities

The Intel® VROC 7.7.0.1273 Release Package is constructed of several components.  The following is the list of those components and their corresponding version numbers.

Due to the components being different entities (but are required for the product to work properly), the component version number may not match and will be different from the package version number.

| Feature | Notes |
| --- | --- |
| Intel UEFI Drivers | - Intel® VROC UEFI Driver version 7.7.0.1054 - VMDVROC_1.efi (HW key enforcement in effect) - Intel® VMD UEFI version 2.7.0.1002 - VMDVROC_2.efi Note: All these images are required and intended to support Intel VMD and Intel VROC (SATA RAID) functionality as a combined installed package. - Intel® VROC (SATA RAID) SATA / sSATA UEFI Driver version 7.7.0.1054 - SataDriver.efi - sSataDriver.efi |
| Intel® VROC Windows Drivers | - Intel® VROC Windows GUI version 7.7.0.1260 - Intel® VROC Windows Installer Package version 7.7_4.0.1 - SetupVROC.exe (Multi-lingual) - Intel® VROC (VMD NVMe RAID) Windows F6 Driver version 7.7.0.1260 Win8 - \iaVROC.free.win8.64bit.7.7.0.1260\iaVROC - Intel® VROC (SATA RAID) Windows F6 Driver version 7.7.0.1260 - Win8 - \iaStorE.free.win8.64bit. 7.7.0.1260 \iaStorE (SATA) - \iaStorE.free.win8.64bit. 7.7.0.1260 \iaStorB (sSATA) - Intel® VROC (NonVMD NVMe RAID) Windows F6 Driver version 7.7.0.1260 Win8 - \iaRNVMe.free.win8.64bit. 7.7.0.1260 \iaRNVMe (NVMe) - Intel VROC CLI version 7.7.0.1260 |
| UEFI Based RAID Configuration Utility | - Intel® VROC version 7.7.0.1054 - RCfgVROC.efi - Intel® VROC SATA / sSATA version 7.7.0.1054 - RCfgSata.efi - RCfgsSata.efi - Note: Secure Boot must be disabled to use this tool |
| UEFI Based Comply Utility | - Intel® VROC version 7.7.0.1054 - RcmpVROC.efi - Intel® VROC SATA / sSATA version 7.7.0.1054 - RCmpSata.efi - RCmpsSata.efi - Note: Secure Boot must be disabled to use this tool |
| UEFI based SATA SGPIO/LED Test utility | - Intel® VROC SATA / sSATA version 7.7.0.1054 - LedToolSata.efi - LedToolsSata.efi - Note: Secure Boot must be disabled to use this tool |
| UEFI based Intel VROC LED Test utility | - Intel® VROC version 7.7.0.1054 - LedToolVROC.efi - Note: This tool can be used to exercise LEDs for NVMe disks behind VMD |
| UEFI Based Clear Metadata Utility | - Intel® VROC SATA / sSATA version 7.7.0.1054 - RClrSata.efi - RClrsSata.efi |
| UEFI Based Intel VROC HW Key Checker | - Intel® VROC Activation Key Checker version 7.7.0.1054 - HWKeyCheckVROC.efi - Note: This tool will check for the presence and type of the HW key |

# Intel VROC Limitations

## Microsoft .NET Framework Removal

As previously described, the Intel VROC product installation application has removed the Microsoft .NET Framework as well as the Intel ASM component.

The following table shows how the removal of the Microsoft .NET Framework may impact the launching of the Intel VROC GUI, based off the Windows operating system installed:

|  | Server 2k12 R2 | Server 2k16 | Windows 2k19 | Windows 2022 |  | Win 10 RS3 | Win 10 RS4 | Win 10 RS5 | Win10 19H1/ 19H2/ 20H1/ 20H2/ 21H1/ 21H2 | Win11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intel VROC 7.7 PV Versions | Install Latest .NET Framework | Install Latest .NET Framework | No Impact | No Impact |  | Install Latest .NET Framework | No Impact | No Impact | No Impact | No Impact |

If the system configuration requires the .NET Framework version to be updated and the system has internet access, a web installer can be used, which should go out and install the latest version.  For example:  (https://support.microsoft.com/en-us/help/4054531/microsoft-net-framework-4-7-2-web-installer-for-windows).

If the system is not connected to the Internet, then an offline version must be downloaded, moved to, and installed on the system.  The following are some additional instruction to help in this process:

- Download the latest version of .NET Framework from Microsoft

- Compress the downloaded image (to avoid potential undesirable side effect as outlined in https://docs.microsoft.com/en-us/dotnet/framework/install/troubleshoot-blocked-installations-and-uninstallations#compat)

- Copy the compressed file to a USB drive

- Copy the compressed file from the USB drive to the Download directory of the platform being configured

- Uncompressed the file

- Run the executable file as administrator

For more information, please refer to https://dotnet.microsoft.com/.

Once the latest version of the .NET Framework is installed, rerun the Intel VROC product installation application.  This helps ensure that all components will start properly.

## Surprise Hot Plug Limitations

Due to Microsoft Windows time restrictions for resuming from S3 and S4, and Intel VMD device identification requirements, Hot Plug of Intel VMD enabled NVMe devices is not supported during S3 and S4 states.

Surprise removal of multiple NVMe SSDs at one time are not supported.  The user must wait until a device is reflected as removed / inserted in device manager for spacing surprise hot plug of Intel VMD enabled PCIe NVMe SSDs in Microsoft Windows.

Due to these limitations, Intel strongly discourages performing Hot Plugs during an S3 power state change.

## Expect Longer Rebuild Times for RAID 5

On a RAID 5 volume, disk cache is being turned off when a volume is degraded. Due to this, the rebuilding times have increased expectedly until the rebuild is completed, and disk cache is enabled again.

This extends to drives being added to a RAID 5 volume as well.

## Intel VROC Command Line Interface

The Intel VROC Command Line Interface (CLI) does not support the RAID Volume name beginning with blank space.

## Intel VROC Trial Version Limitations

Data RAID Only (No Boot Support)

Data RAID must be installed on same make/model of NVMe devices

Once an Intel VROC Upgrade Key has been inserted into the system, the trial version is concluded.  Removing the upgrade key does not re-enable the trial version.  As a result, any existing RAID volumes present while the upgrade key was installed, won’t be seen and could be in an unknown state.

When creating a RAID volume using the Trial version, don’t mix NVMe vendors.  Mixing vendors may result in unexpected behavior.

Please refer to the Intel VROC Trial Version section in the Intel VROC Technical Product Specification for 5.4PV for more details

## Intel VROC PreOS UEFI Driver Uninstall limitations

The Intel VROC UEFI RAID drivers comply with UEFI Specifications for PCI Driver Model for PCI Device Drivers (Section 13.3.3) and may return Status Code “access denied” from UninstallProtocolInterface routine from Boot services (spec. 6.3). This is expected behavior.

## Intel NVMe Wear Leveling Recommendations

NVMe SSD Wear Leveling refers to techniques used to prolong the service life of NVMe drives.  This section outlines recommendations to maximize Wear Leveling on RAID 5 volumes.

| Strip Size No of drives | 4 | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 4 | Optimal | Optimal | Optimal | Optimal | Suboptimal | Suboptimal |
| 5 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 6 | Optimal | Optimal | Optimal | Optimal | Optimal | Suboptimal |
| 7 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 8 | Optimal | Optimal | Optimal | Suboptimal | Suboptimal | Suboptimal |
| 9 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 10 | Optimal | Optimal | Optimal | Optimal | Optimal | Suboptimal |
| 11 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 12 | Optimal | Optimal | Optimal | Optimal | Suboptimal | Suboptimal |
| 13 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 14 | Optimal | Optimal | Optimal | Optimal | Optimal | Suboptimal |
| 15 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 16 | Optimal | Optimal | Suboptimal | Suboptimal | Suboptimal | Suboptimal |
| 17 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 18 | Optimal | Optimal | Optimal | Optimal | Optimal | Suboptimal |
| 19 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 20 | Optimal | Optimal | Optimal | Optimal | Suboptimal | Suboptimal |
| 21 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 22 | Optimal | Optimal | Optimal | Optimal | Optimal | Suboptimal |
| 23 | Optimal | Optimal | Optimal | Optimal | Optimal | Optimal |
| 24 | Optimal | Optimal | Optimal | Suboptimal | Suboptimal | Suboptimal |

Note: It is left to the customer to determine the most effective combination of parameters (number of drives vs. strip size) to achieve their desired performance goals, usage models and drive endurance.

## Must use F6 Install Method

The use of the included Intel VROC F6 drivers are required to install an OS onto an Intel VROC managed device(s).   There is no Microsoft “inbox” driver that supports Intel VROC 7.7.

The supported Microsoft Operating Systems for this product include inbox drivers that support the Intel® C620 and C422 series chipset Platform Controller Hub (PCH) when configured for RAID mode.  It is strongly recommended that the Intel VROC (SATA RAID) F6 drivers included in this release be used instead of the available “inbox” driver.  The provided “inbox” driver is intended only for those customers who may not have the Intel VROC (SATA RAID) F6 drivers readily available and ONLY for installing to a single drive (NOT to a RAID volume).  Once the OS is installed, it is strongly recommended that the Intel VROC 7.7 installer package be installed immediately.  At that point, it will be safe to migrate the SATA system disk into a RAID Volume (using the Intel VROC GUI).

## Intel C620 and C422 series chipset Port Limitations

This limitation is in reference to platforms having a PCH that supports more than 6 SATA ports.  The Intel C620 and C422 series chipset SATA controller supports 8 SATA ports.   As referenced above, The Microsoft Windows Operating systems that contain the “inbox” drivers for the Intel® C620 and C422 series chipset Platform Controller Hub (PCH) when configured for RAID mode, only support 6 ports.  Drives on ports 7 and/or 8 are not enumerated.  For this reason, Intel recommends not using these 2 ports as part of the Windows OS boot installation (as a pass-thru drive or as part of a RAID volume). However, if you do need to use these ports as part of your Windows boot volume, the steps below can be used as a workaround.

Note: you will need a USB drive with the Intel VROC IntelVROCCLI.exe utility.

- After you have created the desired RAID volume that includes ports 7 and/or 8 (which you intend to use as your Windows boot volume) in the PreOS environment, begin the Windows installation process. Make note of the RAID volume name.

- Navigate to the Windows disk selection window.  At this point, select the Load Driver button and install the Intel VROC F6 driver (included in this package).

- Attempt to continue installing the Windows OS onto the RAID volume.  If the installation process does not continue, this error has been encountered.

- Press f10 to invoke a CMD window.

- If you have not already done so, please insert the USB drive into the system.  Navigate to your USB drive with the RstCLI.exe utility.

- Run command: IntelVROCCLI.exe --manage --normal-volume <volumeName>

- This will reset the volume to a normal state.

- Close the CMD window.

- In the Windows disk selection window, reload the Intel VROC f6 driver.

- Once completed, Windows should allow installation on the RAID volume.

## Intel VROC Key Removal/Upgrade Limitation

With Microsoft Windows 10, Fast Startup is enabled by default.  Disable Fast Startup prior to removing/upgrading the Intel VROC HW key. OR, perform a complete reboot when removing/inserting a HW key when Fast Startup is enabled.

## NVMe Port Assignment by Intel VROC

In Windows and UEFI, the port number shown in the Intel VROC interfaces depends on disk enumeration order by the Intel VMD-enabled NVMe driver, which can be different on each platform. The port numbers shown does not reflect the physical PCIe slot. After each hot plug, there is an enumeration process which is NOT fixed.

Please see the Intel® VROC Windows Technical Product Specification for information on the new Intel VROC UEFI Device Info Protocol for unique NVMe physical slot locations.

## Windows 10 RS5/Server 2019

### Idle Power increased

Installing Intel VROC  onto a platform running Windows 10 RS5.  In Windows and UEFI, the port number shown in the Intel VROC interfaces depends on disk enumeration order by the Intel VMD-enabled NVMe driver, which can be different on each platform. The port numbers shown does not reflect the physical PCIe slot. After each hot plug, there is an enumeration process which is NOT fixed.

Please see the Intel® VROC for Windows Technical Product Specification for information on the new Intel VROC UEFI Device Info Protocol for unique NVMe physical slot locations.

### Intel VROC Support for Windows 10 RS5 / Server 2019

Intel RSTe 5.5.0.2013 introduces support for Windows 10 RS5 and Windows Server 2019.

There is a known issue trying to install Windows 10 RS5 / Server 2019.  Installing Windows 10 RS5 or Server 2019 onto an Intel VMD managed device is limited to a single CPU.  For more information please see the Known Issues section below.

This limitation only applies to Intel RSTe 5.5 and earlier releases.  Intel VROC 6.X are not impacted.

## Intel VROC 6.X on Windows Server 2012 R2

When installing Intel VROC 6.X family of products onto Windows Server 2012 R2, the following Microsoft updates must first be applied:

- KB4054566

- KB2999226

- KB2919355

- KB3172729

## Intel VROC (NonVMD NVMe RAID) Upgrade

When attempting to upgrade Intel VROC (NonVMD NVMe RAID), a message may be displayed that an error has occurred trying to disable the virtual device and that the system needs to be rebooted.  If this happens, please reboot and try the upgrade again.

## Intel VMD Support on 8 Socket Platforms

Due to restrictions on NVM Express implementations that reside behind PCI compliant bridges, address space is limited to 32 bit assigned base address registers (for non-prefetchable memory).  This means that there is only 4 Gigabytes of addressable (non-prefetchable) memory space available.  This memory space must be shared between entire platform.  As a result, there is a limitation on the number of Intel VMD controllers that can be enabled (due to these resource constraints).  The following two configurations can be used without encountering memory restriction errors:

- 3 Intel VMDs can be enabled on CPU0 up to CPU6.  Intel VMDs on CPU7 need to remain disabled.  This results in a maximum of 21 Intel VMDs that can be enabled in this configuration.

- No more than two (2) Intel VMDs can be enabled on each CPU in the 8-socket platform. This resulting in a total of 16 Intel VMD domains.

This limitation does not apply to 1, 2 or 4 socket platforms.

## Intel VROC SATA LED Management

When designing a Hot Swap Backplane (HSBP) into a new platform, please make sure that the backplane design supports the platforms External Design Document specification for HSBP support.  Also, please contact your Intel FAE to obtain the latest platform CPLD reference code and information.

The sSATA controller on the Intel Cooper City and Wilson City CRBs have very limited support for using and/or testing backplane management.  Please make sure to review all Intel Cooper City or Wilson City CRB design documentation to understand how the sSATA Controller is laid out.

## Intel VROC Creation Volume Sizes

When Creating a RAID volume, there will be a Volume Size difference seen when comparing a RAID Volume created using the Intel VROC PreOS HII environment and volumes created in the OS.  This has to do with the way the size information is displayed and computed between the two environments.

## MCERR/P_CATERR/Bus Uncorrectable Error with Intel VMD enabled

When CTO occurs, Intel VMD must be chosen to handle these conditions when the root port of the Intel VMD controller that is enabled. This is accomplished by disabling IOMCA on the x16 Intel VMD enabled lanes.

## Intel VROC Legacy Option ROM (OROM)

Intel VROC no longer includes the Legacy OROM in the Intel VROC release packages.  For older platforms, the Intel VROC OROM does not support RAID volumes that are greater than 10 Terabytes.  The Size may not be properly displayed, and Intel has not current plans on resolving this issue.

# Intel VROC Issues

This section outlines the issues reported and internally found that customers need to be aware of.  The issues are broken down into “Known Issues” and “Resolved Issues”.

## Intel VROC Known Issues

This section identifies known Intel VROC issues that are being worked on and are planned to be fixed in a future release.

| id | customer_id | release_notes |
| --- | --- | --- |
| 18015334984 | Internal | Warning Message "Warning: The AHCI driver will be updated to the latest Microsoft AHCI driver in the system" Does Not Occur When Installing Intel VROC in AHCI Mode Using Installer. |
| 1808275753 | Internal | When the system is Rebooting after a Dirty Shutdown, the RAID Volume state may not be properly displayed. This is not Unexpected Behavior after a Dirty Shutdown. |
| 1808094827 | Internal | The Intel VROC (VMD NVMe RAID) PreOS environment may only show 32 NVMe SSDs in the Intel VROC HII. |
| 1508794254 | 22012241826 / 00589273 | Model Number of NVMe device is not able to completely displayed in Device Manager after Intel VMD is enabled - This is a known Limitation |
| 1507753655 | 486129 | When Running Stress Testing on M.2 Slots Managed by the sSATA Controller, an "isStorB" Error may be Logged. - This is expected behavior under such heavy I/O loads. |
| 22012267449 |  | A RAID volume in rebuilding state may state "Bootable: No" in the Intel VROC GUI properties |
| 22011592946 | 00553285 | Intel VROC (SATA RAID) PreOS Health Protocol May not Properly Report the Driver/Controller Information |
| 18016091484 |  | Intel VROC May Not Properly Initiate RAID Rebuild of Matrix (Two) RAID10 Volumes. |
| 18013046256 | Internal | Bad Block Logging during the Intel VROC (SATA RAID) Read Patrol process may Result in Duplicate Logs Being Seen. This is a known limitation with Intel VROC |
| 18012896024 | Internal | Intel VROC GUI May Not Report Proper Number of Media Errors Encountered During Initialization of RAID 5. |
| 18010956435 | Internal | Output May Not Appear In Intel VROC CLI After Attempt To Create RAID 1 Volume From Two Parts of x8 Drive. |
| 14015389340 |  | When using AtaPassThru protocol to get information on attached devices from the Intel VROC PreOS, the value of GetNextTargetLun may not be successful. A work-around is to set the value of "Timeout" field inside "Packet" to a large value (e.g. 3 seconds). |
| 1809400331 | Internal | Intel VROC Rebuild On Hot Insert may not Work Properly on a Matrix RAID Volume when one of the Volumes is a RAID 0. |
| 1805900436 | 401666 | Intel VROC F6 Drivers May Not Properly Load and a Refresh may be Required. |
| 1508749788 | 22012124037 / 00586204 | SATA RAID volume status may not properly display in HII when RAID is degraded with spare drive configured |
| 1407347823 | 266468 | Intel RSTe RCfgRSTeRS.efi Disk IDs may not be consistent with a RAID 5 created with journaling drive and the process may become unresponsive. |
| 18011842936 | Internal | Intel VROC GUI May Not Show "Bootable volume" Status For System RAID Volume. This is a known limiation with the Intel VROC GUI. |
| 18011469063 | Internal | When using the Intel VROC RCfg Tool to Create a RAID Volume, the Warning that All Data Will Be Lost, may not be Displayed. This is a known limitation with this tool. |
| 18011468952 | Internal | When using the Intel VROC HII to Create a RAID Volume using Drives with Different Attributes, the Warning Message may not be Displayed |
| 14011446153 | Internal | Title:Exporting Output, from Intel VROC CLI, to an .xml File may not Work Properly. This is a known limitation with this tool. |
| 18018934449 | Internal | After updating a Wolfpass platform with Intel VROC (VMD NVMe RAID) 7.7 UEFI images, the NVMe RAID submenu may not be available in the UEFI HII. The work around would be to use the RCFG UEFI tool. |

## Resolved Issues in Intel VROC 7.7 PV

| id | customer_id | release_notes |
| --- | --- | --- |
| 18016058101 | 00580596 | Title: Intel VROC Bad Block Management Process May Take (3 to 4 times) Longer than in Previous Versions |
| 18014127243 | Internal | Title: Performing an Intel VROC (SDATA RAID) RAID 5 RAID Write Hole Recovery on a Degraded SATA RAID 5 Volume may not Complete Successfully |
| 14014598311 |  | Title: Intel VROC SATA/sSATA RAID volumes may degrade or fail under high I/O load if a ATA pass through command is issued. |
| 14013794942 |  | Title: The IntelVROCCLi may terminate operation with an error if the user attempts to use the command with -M option, with a drive volume that does not exist. |
| 14013356415 | 00567412 | Title: [CPX-6] Install Protocol Interface failure message after loading VMD UEFI driver |
| 1508964983 | IPS 00600901 | This issue is caused by unsigned iaNullVMD.inf. |
| 1508793548 | IPS 00588770 | Fail to install Windows server 2019 with QWMB CPU |
| 1508747791 | 22012122800 / 00586202 | Title: Hot Removal of a Matrix RAID Member May Result in a System Failure |
| 22011598177 | 00548154 | NVMe drives connected to certain Icelake CPU sku may not be accessible by VROC when VMD is enabled |
| 18016895347 | 00553285 | Title: Intel VROC (SATA RAID) PreOS Health Protocol May not Properly Report the Driver/Controller Information |
| 18015474102 | Internal | Title: Trying to clear a SMART event from a drive after a RAID volume is rebuilt (to another drive), may show up as an Unknown disk. |
| 18014791546 | Internal | Title: Using the Intel VROC CLI Tool to Remove the Metadata on all of the Specified Drives May not Complete Successfully on Drives Identified as "Unknown" |
| 18014524336 | Internal | Title: When using the Intel VROC CLI Tool to Identify the Attached Devices on the SATA/sSATA Controller, Attached ATAPI Devices may not be Properly Reported |
| 18013439721 | Internal | Title: When Running in a Matrix RAID Configuration (Two RAID Volume in a Single RAID Array), the Bad Block Management Process may not Properly Detect Bad Blocks. |
| 18011530136 | Internal | Title: The Intel VROC RCfg Tool may not Properly Display the Warning Message when Trying To Rebuild a Volume to a Drive that is at Least 10% Bigger Than the Largest Member Drive. |
| 1806564424 | Internal | Title: System May Fail to Start After an Unexpected Power Loss |
| 18012842292 | Internal | Title: An Incorrect Error Message ("REQUEST_FAILED: Request is formatted correctly but failed to execute.") May Be Reported By Intel VROC CLI Tool When Trying to Perform a Migration to Unsupported RAID Level. |
| 18012255612 | Internal | Title: The Intel VROC CLI Tool may Return the Wrong Error Message ("INVALID_DEVICE: Request not formatted correctly, device does not exist.") When Trying to Rebuild Degraded RAID Volume To Drive In an Incompatible State. |
| 18012236043 | Internal | Title: Intel VROC CLI May Report Wrong Error ("REQUEST_FAILED: Request is formatted correctly but failed to execute.") When Trying To Remove Member Drive Of System RAID Volume. |
| 18011483476 | Internal | Title: When using the Intel VROC HII to Create a RAID Volume, the Warning Message that All Data on The Member Disks Will Be Lost, may not be Displayed. |
| 1808963497 | Internal | Title: RAID Initialization may not be Automatically Performed when Verify or Verify And Repair is Initiated by the Intel VROC CLI Tool, on an Uninitialized Volume |

## Resolved Issues in Intel VROC 7.6 PV

| Internal Reference Number | IPS Reference Number | Issue Title |
| --- | --- | --- |
| 22012232430 | 00588652 | Using the IntelVROCCLi.exe Tool with the -R Option may not Properly Result in the RAID Volume Rebuild Occurring |
| 14013209937 | 00580596 | Intel VROC Bad Block Management Process May Take (3 to 4 times) Longer than in Previous Versions |
| 1508906750 | 00596519 | [VROC] Drive failure messages seen on initiating IO |
| 14012975200/ 1508768056/ 22011598177 | 00554443/ 00584537/ 00548154 | Not able to install Windows on VROC-managed NVMe or may not be able to access NVMe drives behind VMD with certain Icelake CPU sku. |
| 22011073918 | Internal | A potential silent data loss condition exists in both the Intel VROC (SATA RAID) and the Intel RSTe windows-based products. The potential silent data loss condition exists when the Intel VROC Read Patrol feature is enabled for redundant SATA RAID volumes (RAID1, 5 or 10) and one of the RAID member drives is found to have a Bad Block condition. The Read Patrol process, of using the redundant data to correct the Bad Block, can result in invalid data being written. |
| 1509073224 | Internal | The Intel VROC 7.5 PV Installation application may not properly install the Null Driver to support Device ID 0x09AB. |
| 18016160241 | 600901 | This issue is caused by unsigned iaNullVMD.inf. |
| 14012975200 | 00554443 | NVMe drives connected to certain Icelake CPU sku may not be accessible by VROC when VMD is enabled |
| 1508793548 | IPS 00588770 | Fail to install Windows server 2019 with QWMB CPU |
| 1507753655 | 486129 | When Running Stress Testing on M.2 Slots Managed by the sSATA Controller, an "isStorB" Error may be Logged. - |

## Resolved Issues in Intel VROC 7.5 PV

| Internal Reference Number | IPS Reference Number | Issue Title |
| --- | --- | --- |
| 22011196948 |  | Intel VROC may not properly calculate the LBA locations when performing the Read Patrol Bad Block Recovery Process, Inadvertently missing some Bad Blocks. |
| 22011073918 |  | A potential silent data loss condition exists in both the Intel VROC (SATA RAID) and the Intel RSTe windows-based products. The potential silent data loss condition exists when the Intel VROC Read Patrol feature is enabled for redundant SATA RAID volumes (RAID1, 5 or 10) and one of the RAID member drives is found to have a Bad Block condition. The Read Patrol process, of using the redundant data to correct the Bad Block, can result in invalid data being written. |
| 14012886123 | 00556241 | Intel VROC RAID Write Hole Parity Calculation may be Computed Incorrectly. |
| 22011547837 |  | NVMe drives listed in the VROC BIOS HII page may state incorrect CPU on which it is connected to. Fixed with 7.5 PV |
| 22010691032 | Internal | Intel VROC Installer Help Dialog does not Contain All Available Setup Options. |
| 14011249412 | 00509350 | Activate Led Option May not Work For Non-Intel Drives when Using Intel Only SSD License. |
| 14011049937 | 00247935 | The Order of the SATA Drives Attached to the sSATA Controller May not Match that of how they May be Reported in the SATA Controller |
| 1808275753 | Internal | The Wrong RAID Volume State may be Displayed After a Dirty Shutdown |
| 1507753655 | 486129 | When Running Stress Testing on M.2 Slots Managed by the sSATA Controller, an "isStorB" Error may be Logged. - Expected under such heavy I/O loads. |
| 18012678098 | Internal | When Installing VROC Using Installer With Option "-nodrv" All Drivers Are Uninstalled And No Driver Is Installed In Replacement. This Option May Break Operating System And It Shall Not Be Used. |
| 18011258092 | Internal | Clearing RAID Metadata Using the RCfg Tool may Result in a Platform Hang in UEFI and an ASSERT. |
| 18010905203 | Internal | Creating a RAID Volume in Intel VROC (VMD NVMe RAID) PreOS from Drives Connected through a JBOF may not Succeed and Result in Error Message: "Create volume failed! Cannot write to disk". |
| 1806564424 | Internal | System May Fail to Start After an Unexpected Power Loss |
| 1508007585 |  | VROC HII may display the RAID volume capacity improperly when it is larger than 10000 GB |
| 1507520073 | 2209571335 / 00472167 | Uninstall VROC Windows driver may encounter an error when it is installed and uninstalled multiple times |
| 1306412122 | Internal | Setting LED Configuration of Empty Slot as Fail may not show Fail when slot is empty. |
| 22011382393 | 00543426 | Locate LED may not work properly in VROC HII when using PCIe switch with NPEM support |

## Resolved Issues in Intel VROC 7.0.2 PV

This section identifies those issues that have been resolved in the Intel VROC 7.0.2 PV release.

| Internal Reference Number | IPS Reference Number | Issue Title |
| --- | --- | --- |
| 22011071259 | 00527379 | With Read Patrol Enabled, Recovering from a Bad Block Could Result in a Data Loss Condition. |

## Resolved Issues in Intel VROC 7.0 PV

This section identifies those issues that have been resolved in the Intel VROC 7.0 PV release.

| Internal Reference Number | IPS Reference Number | Issue Title |
| --- | --- | --- |
| 14010261577 | 00470911 | Intel VROC IAStorIcon Scheduled Task may be Inadvertently Removed During a Package Upgrade. |
| 22010314523 | 00493707 | Migration of one RAID10 Volume to a RAID5 Volume May Cause an other RAID10 Volume, in the System, to Automatically Re-Initialize. |
| 18011792455 | Internal | When using a Hot Spare Back Plane from a Whitley Platform in CedarIsland Platform, the SATA LEDs may not Work Properly. - Not an Intel VROC Issue. |
| 14010852023 | 00492469 | Intel VROC HII may not Display RAID Volume Information Correctly When the RAID Volume is Greater than 10 Terabytes. |
| 1808514953 | Internal | Booting a Platform with 48 NVMe Drives may Result in a System Failure. |
| 1808389290 | Internal | The Intel VROC Rebuild LED behavior may not operate properly when Customized to blink all LEDs during a RAID rebuild |
| 1806994368 | Internal | Performing Hot Plug drive replacement on a degraded RAID volume, with Rebuild on Hot Insert enabled, may not properly initiate an automatic RAID rebuild. |
| 1507501583 | Internal | When Resuming From and S3 Power State Change, the "System" Label may not be Displayed Properly in the Intel VROC GUI. |
| 1409667894 | 448534 | LED Locate from HII BIOS VROC Menu Causes Page to Exit Prematurely |
| 1808703820 | Internal | When Using the Intel VROC CLI Tool to Adding a Drive to an Existing Array, the Volume Strip Size may no be Displayed Properly. |
| 1808580403 | Internal | When Performing a Drive Hot Plug, the Event Logger may Show Other Drives (Not Involved in the Hot Plug) having Encountered Hot Plug Events |
| 1807170210 | Internal | An Intel VROC GUI Volume Creation Warnings Message May Overlay ontop of a Submenu |
| 1806564426 | Internal | Event Lot May Not Poroperly Show "RAID volume {VolumeName} is normal" Message after a Rebuild Completes |
| 18011690000 | Internal | Activate LED Option may not Work in VROC CLI and UI, When Using Intel-SSD-Only License. - Internal Testing Issue |

## Resolved Issues in Intel VROC 6.3 PV

This section identifies those issues that have been resolved in the Intel VROC 6.3 PV release.

| Internal Reference Number | IPS Reference Number | Issue Title |
| --- | --- | --- |
| 14010267807 | 2209520253 / 00471069 | Performing a Drive Surprise Hot Remove from a Matrix RAID Array May Result in a Platform Failure. |
| 1507649523 | IPS 00482474 | Using the CC_CSMI_SAS_GET_DRIVER_INFO Command May Not Report Correct Drive and Driver information. |
| 18010792702 | 00470911 | Intel VROC IAStorIcon Scheduled Task may be Inadvertently Removed During a Package Upgrade. |
| 1809577436 |  | An Intel VROC spanned RAID10 Volume may not Start Rebuilding Automatically after a Rebuild on Hot Insert Condition. |
| 1806397184 | Internal | IIntel RSTe NVMe Pre-Purley Platform with 48 NVMe Drives and Max Volumes. Degraded RAID Volume May Encounter a System Failure While Booting |
| 1507369786 | Internal | Migrating a single NVMe boot drive to an Intel VROC (VMD NVMe RAID) Volume and Performan a System Sleep Power State Change May Result in a System Crash |
| 1806411891 | Internal | RAID Volume May Become Degraded After Reboot |
| 1806397164 | Internal | Intel RSTe NVMe Pre-Purley Platform with 48 NVMe Drives and 24 Volumes May Encounter a boot Failure |
| 1507522222 | 2209595864 / 00473612 | Intel VROC LED Management may Illuminate the Locate LED Option During Another RAID Operation and may not stop when the Operation Completes. |
| 1806930160 | Internal | An Error message in event viewer “The driver detected a controller error..” may be displayed when performing platform power state changes with Intel VROC installed. |
| 1806419240 | Internal | Intel VROC (VMD RAID) NVMe Drive May be Marked as Available After Removal |

## Resolved Issue in Intel VROC 6.2 PV

This section identifies those issues that have been resolved in the Intel VROC 6.2 PV release.

| id | customer_id | release_notes |
| --- | --- | --- |
| 1606900429 | 405188 | System Crash May Occurred when load Intel VROC (VMD NVMe RAID) F6 driver while the Intel VROC RAID is Under an Initialize State. |
| 1409230595 | 424290 | Intel VROC UEFI May Not Properly Report the VMD Controller BDF in the HII |
| 1409017125 | 416904 | "UEFI Driver Hangs When the Metadata is Malformed" |
| 1408968353 | 404672 | Intel VROC (VMD NVMe RAID) UEFI HII Menu May Cause the BIOS Setup Menu to be Improperly Displayed |
| 1409194760 | 424543 | Activity LED is not blinking when SATA drive in RAID mode |
| 1808452676 | Internal | A system, with the OS installed onto a RAID volume, may not properly resume after several Hybrid Sleeps |
| 1808377588 | Internal | A system running Intel VROC, with the OS installed onto a RAID volume, may not properly resume from a Hybrid Sleep state, after performing multiple Hybrid Sleeps. |
| 1808060543 | Internal | The Intel VROC 6.2 UWD UI Application may not properly function on a platform with Intel VROC 6.0 driver package. Please make sure that when using the Intel VROC UWD UI Application, that the UI version matches the driver package (i.e. both should be Intel VROC 6.0 or both should be Intel VROC 6.2) |
| 1807345165 | Internal | The CLI Tool May Not Properly Expand Existing RAID Volumes |
| 1807107325 | Internal | When using the Intel VROC CLI Tool to Create RAID Volumes One of the Disks May Show an Disk Size of 0 GB after the Volume Creation Completes. |
| 1806677977 | Internal | Bad Blocks May Not be Properly Reported in a RAID 5 Volume |
| 1806534894 | Internal | The Intel VROC CLI Tool May Allow Data Migration With a Smaller Drive |
| 1806503629 | Internal | Creating a RAID 1 Volume from an Existing Drive May Result in a Failed RAID Volume |
| 1507222187 | 438747 | VROC icon is missing in system tray Issue description: VROC GUI icon is not exist in system tray in Windows10. |
| 1409784946 | 455468 | Hot Removal of a RAID 5 RAID Write Hole (RWH) Journaling Drive may cause the platform to become unstable and may cause a system failure System Instability and BSODs |
| 1409371408 | Internal | Intel VROC GUI May Require the "Proceed with deleting data" Box be Selected when not Expected. |
| 1407853994 | 325087 | Degraded SATA RAID 5 may not boot if disk on SATA controller port 0 is removed or fails Waiting for 6.2.0.1108 confirmation from HP. |
| 1407801045 | 325604 | The Intel VROC RWH Policy May inadvertently Change from Journaling to Distributed if one member drive is missing |
| 1407219909 | 246717 | New VMDVROC_1.efi / VMDVROC_2.efi driver will increase boot time around 4 seconds |
| 1806782204 | 429264 | Intel VROC GUI may not Properly Open Unless "Run as Administrator" |
| 1506398660 | 281938 | RAID10, hot-plug two member disks, re-plugged second disk can't rebuilding. |
| 1409584095 | 445268 | Event Log Error 4156/4155 Seen during stress testing |
| 1807977956 | Internal | The platform may encounter a system failure as a result of performing a Hybrid Sleep cycle on an Intel VROC RAID5 Volume (the first Hybrid Sleep after running the Intel VROC installation application). |
| 1807962656 | Internal | The Intel VROC PreOS UEFI may not properly display the full serial number of a removed/offline Volume member drive in the UEFI Health Protocol information. |
| 1807158496 | Internal | The Intel VROC RWH Policy May Change from Journaling to Distributed after a Drive Hot Unplug |
| 1806564409 | Internal | Platform May Not Properly Boot After a Dirty Shutdown with I/O on a RAID 4 volume (RWH Distributed) |